# Background: Radiation Effects on Digital Electronics and FPGA-Based Instrumentation

The deployment of field-programmable gate arrays (FPGAs) in nuclear instrumentation and control (I&C) systems represents a convergence of two historically separate engineering disciplines: the radiation effects community, which has characterized and mitigated the response of microelectronics to ionizing radiation primarily in the context of space and defense applications, and the nuclear industry, which is confronting the simultaneous pressures of fleet aging, digital modernization, and cost reduction described in Chapter 1. This chapter provides the technical background required to understand the FPGA-based monitoring systems developed in Chapters [ch:digital_electronics] and [chap:fpga_benchmark]. It begins with the radiation environments of concern, then develops the physical mechanisms by which ionizing particles damage or disturb microelectronic devices, surveys the taxonomy of single-event effects (SEEs) with emphasis on those most relevant to SRAM-based FPGAs, describes the fault-tolerance architectures used to mitigate those effects, and concludes with the testing methodologies used to characterize and qualify devices for radiation environments.

## Radiation Environments for Digital Electronics

Digital electronics encounter ionizing radiation in three principal deployment contexts relevant to this work: the near-Earth space environment, the nuclear reactor plant environment, and the terrestrial cosmic-ray environment at ground level. Each environment differs in particle species, energy spectra, and dose rates, and each imposes distinct qualification requirements.

### The Space Radiation Environment

The space environment exposes electronics to three primary sources of energetic particles. Trapped protons and electrons in the Van Allen belts (primarily 1–10 MeV protons in the inner belt, up to several hundred MeV in the South Atlantic Anomaly) produce continuous total ionizing dose accumulation and displacement damage that degrades device performance over mission lifetime . Galactic cosmic rays (GCRs) are fully ionized nuclei spanning the entire periodic table, with energies from tens of MeV/nucleon to above 10​20 eV; they are isotropic, difficult to shield, and responsible for the bulk of single-event effects at high altitude . Solar energetic particles (SEPs), produced during coronal mass ejections and solar flares, deliver intense but episodic fluences of protons (up to hundreds of MeV) and heavy ions that can cause acute upsets far exceeding the GCR background .

The cumulative radiation dose received by a spacecraft depends strongly on orbit, shielding, and mission duration. Low-Earth orbit (LEO) satellites at International Space Station altitude (∼400 km, 51.6​∘ inclination) accumulate roughly 5–20 krad(Si)/year behind 100 mil aluminum shielding, while geostationary (GEO) and medium-Earth orbit (MEO) satellites may see 30–100 krad(Si)/year due to prolonged residence in the Van Allen belts . Qualification standards such as MIL-STD-883 and ESA ESCC 22900 require demonstration of margin over these mission doses before flight hardware can be approved.

### The Nuclear Plant Radiation Environment

Within an operating nuclear power plant, electronics installed in or near the reactor building are exposed to a mixed neutron-gamma field whose intensity depends strongly on location. In-core instrumentation must withstand integrated fast-neutron fluences of 1017–1019 n/cm​2 and total gamma doses of 106–108 rad over the reactor lifetime. Ex-core instrumentation in the primary containment sees lower but still significant fluences of 1013–1016 n/cm​2 and gamma doses of 104–106 rad . Safety I&C systems in the auxiliary building and control room operate at much lower dose rates (typically well below 1 rad/year) but must maintain functionality under design-basis accident conditions that could spike the local dose rate by several orders of magnitude for hours to days .

The neutron fluence in reactor environments produces displacement damage in semiconductor lattices that is qualitatively distinct from ionizing dose effects: energetic neutrons displace silicon atoms from lattice sites, creating vacancy-interstitial pairs and vacancy clusters that act as generation-recombination centers, increase leakage current, reduce minority carrier lifetime, and degrade transistor gain . SRAM-based FPGAs, whose configuration memory cells rely on stable stored charge, are particularly sensitive to neutron-induced displacement damage in the pass transistors that gate cell access.

### The Terrestrial Cosmic-Ray Environment

At sea level, the secondary cosmic-ray flux consists primarily of muons, with a smaller but device-relevant component of high-energy neutrons (“spallation” neutrons) produced by hadronic showers in the upper atmosphere. The terrestrial fast-neutron flux at sea level is approximately 1.3×10−2 n/cm​2/s for neutrons above 10 MeV, increasing by roughly a factor of three per kilometer of altitude . Although this flux is orders of magnitude lower than in accelerator or reactor environments, it is ubiquitous and non-negligible for high-density memories operating in safety-critical ground applications. The JEDEC standard JESD89A quantifies the terrestrial soft-error rate environment and defines the reference neutron spectrum at sea level against which device qualification measurements are normalized .

## Total Ionizing Dose Effects

Total ionizing dose (TID) refers to the cumulative energy deposited in a material by ionizing radiation, principally energetic electrons, protons, and gamma photons, integrated over the operating lifetime of the device. In metal-oxide-semiconductor (MOS) structures, which form the transistor basis of all modern digital FPGAs, the primary site of TID damage is the gate oxide and the field oxide used to isolate adjacent transistors.

### Charge Trapping in Gate Oxides

When ionizing radiation passes through a SiO​2 gate oxide, it generates electron-hole pairs with a yield proportional to the local electric field and the oxide thickness. Electrons, which have high mobility in SiO​2 (μe≈20 cm​2/V⋅s), sweep out of the oxide rapidly under the applied gate field on nanosecond timescales. Holes, with a mobility orders of magnitude lower, undergo a dispersive hopping transport toward the Si/SiO​2 interface, where a fraction become trapped at pre-existing or radiation-induced oxide trap sites . The resulting positive oxide trapped charge shifts the transistor threshold voltage VT in the negative direction (for n-channel MOSFETs), eventually causing the device to conduct when it should be off, a failure mode known as threshold-voltage rolloff .

In parallel, radiation generates interface trap states at the Si/SiO​2 boundary by breaking Si-H bonds. These interface traps contribute both a fixed charge and a frequency-dependent recombination current that degrades transistor transconductance, increases 1/f noise, and reduces drive current. Unlike oxide trapped charge, which anneals partially at room temperature, interface traps are thermally stable on device-lifetime timescales .

### TID Effects in Modern FPGAs

The susceptibility of a CMOS process to TID is strongly dependent on gate oxide thickness: thinner oxides, which dominate in sub-micron nodes, trap fewer holes because the thin-oxide regime favors rapid recombination before hole transport can occur. A 65-nm process FPGA may tolerate 300–500 krad(Si) before functional failure, compared to 30–100 krad(Si) for older 1-μm process devices . However, the isolation structures (shallow trench isolation, STI) in modern processes are relatively thick and remain a TID-sensitive element, causing leakage current between adjacent transistors at doses that do not yet affect the gate oxides.

For the nuclear instrumentation context, the TID tolerance of commercial SRAM FPGAs in the control room or auxiliary building is generally adequate without special hardening, as cumulative doses over a plant lifetime at those locations are well below the few-krad level . The primary concern for FPGAs in nuclear plants is therefore not TID but the stochastic single-event effects described in the next section, which occur at arbitrarily low dose rates and cannot be mitigated by simply limiting exposure time.

## Single-Event Effects: Physical Mechanisms

### Ion Track Formation and Charge Collection

A single-event effect (SEE) is a functional or physical disturbance in a microelectronic device caused by the passage of a single energetic particle. The mechanism begins with the energy deposition of the incident particle along its track through the semiconductor. Charged particles, heavy ions, protons, and the recoil nuclei from neutron elastic scattering or nuclear reactions, lose energy primarily through Coulomb interactions with the semiconductor electrons, creating a dense column of electron-hole plasma along the track.

The key parameter governing SEE sensitivity is the linear energy transfer (LET), defined as the energy deposited per unit path length:

LET=−1ρdEdx MeV⋅cm2/mg,

where ρ is the material density. For a silicon target, an LET of 1 MeV⋅cm​2/mg deposits approximately 1.6×10−13 J per micron of path length, producing roughly 10​4 electron-hole pairs per micron. The LET of a given ion depends on its atomic number Z, mass A, and kinetic energy; heavy ions near the middle of the periodic table (e.g., Fe, Kr, Xe, Bi) with energies of tens of MeV/nucleon achieve the highest LETs, up to ∼100 MeV⋅cm​2/mg in silicon .

Following the primary ionization event, the excess carriers are collected at the nearest reverse-biased junction by two mechanisms illustrated in Figs. 1.1 and 1.2. Drift collection occurs because the high carrier density temporarily collapses the depletion region and extends the electric field deep into the substrate, a phenomenon known as funneling, sweeping charge toward the junction on picosecond timescales . Diffusion collection then operates on slower nanosecond timescales as carriers generated outside the funnel diffuse through the neutral substrate and are eventually collected. The total collected charge Qcoll is determined by the integral of the current transient at the junction and must exceed the critical charge Qcrit of the struck node to cause a SEE.

Simulated electron concentration during the funneling phase following a heavy-ion strike in a reverse-biased n​+/p junction.

Electrostatic potential distribution during funneling, showing the field distortion caused by the ion track .

### The SEE Sensitivity Figure of Merit: Cross-Section

The probability that a particle causes a SEE is characterized by the device SEE cross-section σ, defined as the number of events observed per unit incident particle fluence:

σ(L)=NeventsΦ cm2/device or cm2/bit,

where Φ is the fluence in particles/cm​2. The cross-section is measured as a function of LET or proton energy and typically exhibits a threshold below which no events are observed and a plateau at high LET where every ion strike within the sensitive volume causes an event. The threshold LET (LET​th) and the saturation cross-section (σsat) together characterize the inherent hardness of the device .

The on-orbit SEE rate is computed by convolving the measured cross-section curve with the environment’s differential LET spectrum, a calculation codified in tools such as CREME96 and OMERE . For a device with σsat=10−6 cm​2/bit and LET​th=3 MeV⋅cm​2/mg in a LEO orbit, the predicted upset rate is typically 10−5–10−3 upsets/bit/day, meaning a 16-Mbit configuration memory will experience roughly 10–1,000 upsets per day without mitigation.

## Single-Event Effects Taxonomy

SEEs span a range of severity from fully recoverable bit flips to permanent device destruction. Fig. 1.3 presents the standard SEE taxonomy tree, which organizes effects by their recoverability and the circuit element affected.

Taxonomy of single-event effects in microelectronic devices, organized by recoverability (soft vs. hard) and affected circuit element. Effects most relevant to SRAM-based FPGA operation are shown in bold.

### Single-Event Upset (SEU)

A single-event upset (SEU) is a non-destructive, recoverable change in the logic state of a bistable storage element, a flip-flop, SRAM cell, or register, caused by charge collection from a single particle strike. The struck node’s voltage is transiently driven past the switching threshold of the latch, causing it to adopt the opposite state . In an SRAM-based FPGA, the configuration memory that encodes the user’s design is implemented as an array of SRAM cells; an SEU in this array changes the programmed function of a logic element, routing multiplexer, or interconnect switch, potentially altering the circuit’s behavior in ways that are not immediately apparent without continuous monitoring .

The critical charge for SEU in a modern 28-nm SRAM cell is on the order of 1–10 fC, achievable by the recoil nucleus from a proton nuclear interaction or by a moderate-LET heavy ion . As process nodes have scaled, Qcrit has decreased roughly proportionally with feature size, partially offset by reductions in node capacitance and improvements in cell drive strength.

### Multiple Cell Upset (MCU)

A multiple cell upset (MCU) occurs when a single particle deposits sufficient charge to simultaneously flip two or more adjacent memory cells. MCUs are of particular concern because they can defeat single-bit error correction codes (SECDED) that are effective against isolated SEUs. The probability of MCU scales with LET and is strongly geometry-dependent: cells laid out in adjacent rows and columns of the SRAM array are co-vulnerable, while cells interleaved across different physical locations are not . In modern planar CMOS processes, MCU events can involve tens of cells from a single high-LET heavy-ion strike, while proton-induced MCUs typically involve two to four cells from the recoil of a proton nuclear reaction .

### Single-Event Transient (SET)

A single-event transient (SET) is a voltage glitch on a combinational logic node caused by charge collection at a circuit junction. Unlike an SEU, which corrupts a storage element, an SET propagates through combinational logic and may or may not be captured by a downstream flip-flop, depending on the timing relationship between the transient and the clock edge. Fig. 1.4 illustrates the generation of an SET at an inverter output, and Fig. 1.5 shows the conditions under which an SET propagating through a logic cone is captured by a flip-flop.

Generation of a single-event transient (SET) at a CMOS inverter output, showing ion-track charge deposition at a reverse-biased drain junction .

SET propagation through a combinational logic cone and arrival at a flip-flop data input, showing the timing relationship with the clock edge .

In high-speed digital logic, SET pulse widths of tens to hundreds of picoseconds are comparable to clock periods, making SET-induced SEUs a dominant error mechanism in modern FPGAs operating above 100 MHz . At the lower clock rates typical of nuclear I&C applications, SET capture probability is reduced, but long transients from high-LET events remain a concern.

### Single-Event Functional Interrupt (SEFI)

A single-event functional interrupt (SEFI) is an SEU in a control register, finite state machine, or configuration memory that causes a device to enter an erroneous operating mode, resetting itself, locking up, or outputting invalid data, that requires a system-level reset to clear. In FPGAs, SEFIs in the configuration controller or global control registers can cause a partial or complete loss of the user’s programmed logic, a far more serious consequence than a single flipped bit in the data path . SEFI rates are typically much lower than SEU rates per bit (because control registers represent a small fraction of total bits), but their operational consequence is disproportionately severe.

### Single-Event Latchup (SEL)

Single-event latchup (SEL) is a potentially destructive condition in which a particle strike triggers the parasitic p-n-p-n thyristor inherent in bulk CMOS processes, causing a low-impedance path to form between the supply rails. If not cleared by power cycling within milliseconds, the resulting current can permanently destroy the device through thermal runaway . SEL susceptibility is strongly suppressed in silicon-on-insulator (SOI) processes (which lack the bulk substrate needed for the parasitic thyristor) and by epitaxial substrates with heavily doped buried layers. Modern commercial SRAM FPGAs fabricated in bulk CMOS are SEL-susceptible and require current-limiting power supply protection and power-cycling circuitry in space and nuclear applications.

### Single-Event Burnout (SEB) and Gate Rupture (SEGR)

Single-event burnout (SEB) and single-event gate rupture (SEGR) are catastrophic destructive effects confined primarily to power MOSFETs and bipolar transistors operating at high drain-source voltages. SEB occurs when a heavy-ion strike in the epitaxial drain region of an OFF-state power MOSFET triggers a parasitic n-p-n transistor, causing sustained high-current conduction that thermally destroys the device . SEGR results from the momentary field enhancement in the gate oxide caused by the ion-track plasma, which can exceed the oxide breakdown field and permanently rupture the gate dielectric. Figs. 1.6, 1.7, and 1.8 illustrate the cross-sectional geometry and onset conditions for these effects.

Cross-sectional schematic of single-event burnout (SEB) in an n-channel power MOSFET, showing the heavy-ion track (dashed), epitaxial drain, and parasitic n-p-n transistor .

Schematic of the single-event gate rupture (SEGR) mechanism, showing the ion-track plasma and gate-oxide field region .

Measured SEGR onset VGS as a function of heavy-ion LET for several gate-metal species .

SEB and SEGR are not directly applicable to logic-level SRAM FPGAs, which operate at supply voltages far below the breakdown thresholds of their internal power transistors. They are, however, relevant to the power supply circuitry and gate drivers that support an FPGA-based system, and must be considered at the system level in any radiation-hardened design.

## FPGA Architecture and Radiation Vulnerability

### SRAM-Based FPGA Architecture

Modern high-density FPGAs are organized as a two-dimensional array of configurable logic blocks (CLBs), interconnected by a programmable routing fabric and surrounded by dedicated I/O, clock management, block RAM, DSP, and hard-IP peripherals. The entire programmable function of the device is encoded in a configuration memory bitstream stored in an on-chip SRAM array, which for large devices may contain 50–500 million configuration bits . The configuration SRAM is loaded from external non-volatile storage (typically QSPI flash or EEPROM) on power-up and retained in SRAM for the duration of operation.

This architecture has a fundamental radiation vulnerability: every bit of the configuration SRAM is continuously susceptible to SEU throughout device operation. An SEU in the configuration memory can alter the function of a lookup table (LUT), change the routing of a signal, enable or disable a flip-flop reset, or modify the address decode of a block RAM, depending on which configuration bit is struck. Unlike an SEU in a user-data register, which affects only the current computation, a configuration memory SEU silently corrupts the device’s programmed behavior until it is detected and corrected .

The sensitivity of different FPGA configuration bit types to functional corruption varies substantially. Bits that control LUT contents are always functionally significant; bits that control unused routing resources may have no effect. Studies of Xilinx Virtex and Kintex FPGAs have found that roughly 15–30% of configuration bits are “essential”, their upset causes a detectable output error, with the remainder corresponding to unused or redundant resources . This essential bit fraction is critical for predicting the functional upset rate from a measured raw upset rate.

### Radiation Sensitivity Across FPGA Generations

Quinn et al. surveyed the radiation reliability concerns of Xilinx FPGAs from the Virtex through Virtex-5 generations, finding that each new process node brought reduced Qcrit (worsening raw SEU sensitivity per bit) but also reduced bit count per logic function (improving functional upset rate per implemented operation) . SRAM cell cross-sections measured at particle accelerators for 90-nm-class devices (Virtex-4/5) are typically in the range of 10−14–10−13 cm​2/bit at high LET, corresponding to functional upset rates of 10−3–10−2 upsets per device per day in a representative LEO orbit. Rakhman et al. characterized the SRAM configuration cells of a Xilinx Kintex-7 FPGA under both heavy-ion and proton irradiation, finding a saturation cross-section of approximately 5×10−14 cm​2/bit and a proton cross-section of ∼10−15 cm​2/bit at 200 MeV .

For nuclear plant environments, the neutron-induced upset rate at control-room locations (fast-neutron flux <103 n/cm​2/s) is orders of magnitude lower than in space and may be further reduced by modest concrete shielding. However, design-basis accident scenarios that expose I&C systems to elevated fluences over hours to days require that the system either be immune to configuration upsets or be capable of detecting and correcting them within the plant’s safety response time.

## Fault-Tolerant Mitigation Techniques

### Triple Modular Redundancy

The foundational technique for SEU mitigation in digital systems is triple modular redundancy (TMR), originally proposed by Lyons and Vanderkulk in 1962 for improving the reliability of vacuum-tube computers . In TMR, the protected function is replicated three times, and a majority voter selects the output of the two (or three) agreeing copies. A single SEU in any one replica is masked because the two healthy copies outvote the corrupted one, and the voter output remains correct. The technique requires the voter itself to be robust, either because it is implemented in radiation-hard technology or because the voter is a simple gate whose critical charge greatly exceeds that of the storage elements it protects.

Overview of fault-tolerant mechanisms for SRAM-based FPGAs, ranging from single-level flip-flop TMR to full-system architectural redundancy .

Voter logic flowchart for a TMR system, showing majority voting across three independent replicas .

In FPGA implementations, TMR is applied at multiple granularities. Full TMR triplicates every logic element, flip-flop, and routing resource in the design, providing maximum fault coverage at the cost of roughly 3× resource usage and power. Partial TMR applies triplication only to the most safety-critical paths, reducing overhead while protecting against the most consequential upsets . Register-file TMR protects only the storage elements (flip-flops and block RAMs) while leaving combinational logic unprotected, trading reduced area for acceptance of transient SET-induced errors that are not captured . Fig. 1.9 illustrates these trade-offs, and Fig. 1.10 shows the voter logic in detail.

The TMR approach is well-established in space applications and has been demonstrated to reduce functional upset rates by approximately two orders of magnitude in Xilinx Virtex-class FPGAs at representative GCR fluences . For nuclear I&C applications, Sciber et al. demonstrated a TMR-protected FPGA-based safety system for a research reactor that maintained correct output through simulated configuration upsets at a rate 500× the expected plant environment level .

### Configuration Scrubbing

TMR corrects the voter output but does not repair the configuration memory of the corrupted replica. After a sufficient number of upsets accumulate, two replicas may be simultaneously corrupted, defeating majority voting. Configuration scrubbing addresses this by periodically refreshing the FPGA’s configuration memory from a known-good reference stored in non-volatile memory .

In external scrubbing, an external processor reads the current configuration bitstream from the FPGA, compares it bit-by-bit with the golden reference, and writes back corrected frames where differences are detected. This approach can detect and correct errors at rates determined by the scrub period, which must be short compared to the mean time between double upsets in the TMR replicas . For a device with 107 configuration bits, a bit-upset rate of 10−3 upsets/bit/day, and a required double-upset probability below 10−6 per mission, the required scrub period is on the order of seconds, achievable with standard scrubbing controllers operating at JTAG bitstream rates .

Partial reconfiguration scrubbing uses the FPGA vendor’s partial reconfiguration infrastructure to overwrite individual configuration frames without resetting the entire device, allowing scrubbing to proceed concurrently with normal device operation . Xilinx Virtex/UltraScale and Intel Stratix devices both support partial reconfiguration, and open-source scrubbing controllers (e.g., the Scrub Engine developed at Brigham Young University) implement this approach . Herrera-Alzu and López-Vallejo systematically characterized the scrubber performance for Xilinx Virtex-5 FPGAs, demonstrating that scrub periods below 50 ms reduced the effective functional upset rate below 10−6 upsets/day at LEO conditions .

### Combined TMR-Scrubbing and Hybrid Approaches

For the highest reliability requirements, TMR and scrubbing are deployed together: TMR provides instant correction of single upsets, while scrubbing prevents the accumulation of uncorrected errors that would eventually defeat TMR . The combination reduces the effective uncorrectable upset rate to the probability of two upsets occurring within a single scrub period before the first is corrected, typically several orders of magnitude lower than either technique alone.

Hybrid approaches extend these concepts to the system level. Stoddard et al. demonstrated a configuration in which an FPGA running a spacecraft attitude control algorithm was protected by TMR at the register level, partial-reconfiguration scrubbing at the configuration level, and watchdog-timer reset at the system level, achieving an estimated mean time between failures of >105 hours in a GEO radiation environment . The architecture developed in Chapters [ch:digital_electronics] and [chap:fpga_benchmark] for nuclear plant monitoring adopts a similar layered approach, with TMR protecting the safety logic, periodic configuration scrubbing maintaining the configuration integrity, and a redundant processor monitoring the voter-disagreement signal to detect any uncorrected upset before it propagates to a safety output.

### Error Detection and Correction in Block RAMs

Block RAMs (BRAMs) in FPGAs store application data, not configuration bits, and are equally susceptible to SEUs. Xilinx and Intel FPGAs provide optional built-in single-error correction, double-error detection (SECDED) Hamming codes for BRAM data, which correct single-bit upsets transparently and flag double-bit upsets as uncorrectable . For nuclear applications where the BRAM stores measurement data or plant state variables, SECDED with periodic scrubbing provides an acceptable level of protection, while dual-port BRAM with cross-checking between two separate memory arrays provides an additional layer of protection against MCU events that span multiple bits within the same Hamming word.

## COTS FPGAs in Nuclear and Radiation-Critical Applications

### The Cost-Performance Trade-off

Purpose-built radiation-hardened FPGAs, such as the Microchip RTG4 (130-nm flash-based, inherently SEU-immune configuration) and the AMD/Xilinx XQRKU060 (16-nm FinFET, rad-hard by design), provide guaranteed TID tolerance exceeding 300 krad(Si) and quantified SEL immunity, but at unit prices of $50,000–$500,000 and with logic densities one to two generations behind commercial equivalents . For nuclear plant I&C modernization programs that may require hundreds of processing nodes, this cost premium is prohibitive.

Commercial off-the-shelf (COTS) FPGAs from the Xilinx 7-series, UltraScale, and Intel Cyclone/Arria families offer 50–100× lower cost per gate, higher logic density, and access to modern soft-IP ecosystems, at the cost of no radiation-hardness guarantee from the manufacturer . The engineering question is therefore not whether COTS FPGAs are radiation-tolerant, but whether their radiation response, combined with appropriate mitigation, can be characterized to a level of assurance sufficient for the safety classification of the application.

### Demonstrated COTS FPGA Performance in Radiation Environments

Several accelerator and nuclear programs have demonstrated that COTS FPGAs with TMR and scrubbing can meet stringent reliability requirements at radiation levels consistent with nuclear plant I&C. Lee et al. irradiated a Xilinx Kintex-7 FPGA (28-nm) with protons at TRIUMF and with neutrons at Los Alamos National Laboratory’s Weapons Neutron Research facility, measuring a total accumulated dose of 300 krad(Si) with no functional failure, and recording zero configuration upsets attributable to TID at that dose level . Wirthlin et al. demonstrated a Leon3 soft-core processor implemented on a Kintex-7 FPGA that maintained correct program execution through 3×1010 n/cm​2 of 14-MeV neutron irradiation with TMR+scrubbing active, corresponding to a ground-level exposure equivalent of several decades of control-room operation .

The CERN high-energy physics community has deployed COTS FPGAs extensively in the LHC detector electronics, where annual doses in some locations reach tens of krad and the SEE environment from secondary shower particles is comparable to LEO. Systematic characterization programs at CERN show that 28-nm Kintex-7 and UltraScale devices exhibit no destructive effects (SEL, SEB, SEGR) at bias voltages within datasheet ratings at LET up to 62 MeV⋅cm​2/mg, and that TMR+scrubbing reduces functional upset rates below the 10−9/hour target imposed by LHC safety systems .

Pignol reviewed the use of COTS components in safety-critical space avionics and concluded that the combination of architectural mitigation (TMR, watchdog), physical mitigation (shielding, latchup current limiters), and comprehensive testing is sufficient to achieve failure-in-time rates competitive with purpose-built rad-hard parts, at substantially lower cost . The same conclusion drives the DOE GEMINA program, which targets AI/ML-enabled COTS digital systems for advanced reactor I&C , and the ANS-recommended roadmap for I&C modernization at the existing LWR fleet .

### Regulatory Pathway for COTS FPGAs in Nuclear Plants

The NRC’s regulatory framework for digital I&C in nuclear plants (10 CFR 50 Appendix A, GDC 21–25; Regulatory Guide 1.152; NUREG/CR-6263) does not prohibit COTS devices but requires that the licensee demonstrate that the device meets the qualification requirements for its safety classification, including radiation qualification per IEEE Std 7-4.3.2 and post-accident monitoring per RG 1.97 . The key qualification documentation for an FPGA-based safety function includes: (1) a radiation characterization data package demonstrating TID tolerance, SEL immunity, and SEU cross-section measurement; (2) a functional upset rate prediction for the plant-specific radiation environment; (3) a mitigation architecture description showing that the functional upset rate with TMR+scrubbing meets the specified single failure criterion; and (4) a test program demonstrating correct operation under the combined effects of irradiation and functional loading .

## Testing Methodologies for FPGA Radiation Qualification

The qualification of FPGAs for radiation environments requires experimental measurement of both TID and SEE parameters, since analytical prediction from process parameters alone is not sufficiently accurate for safety-relevant applications. Fig. 1.11 illustrates the principal testing approaches and their relationship to the environment being simulated.

Summary of testing approaches for radiation hardness assurance of digital electronics, organized by radiation source, simulated environment, and measured effect .

### Heavy-Ion Testing

Heavy-ion irradiation at particle accelerators is the primary technique for measuring SEE cross-sections as a function of LET and for identifying the SEL, SEB, and SEGR thresholds of a device. Facilities commonly used for FPGA radiation testing include the Brookhaven National Laboratory Tandem Van de Graaff (LET range 1–100 MeV⋅cm​2/mg), the Lawrence Berkeley National Laboratory 88-Inch Cyclotron (LET up to 85 MeV⋅cm​2/mg), the Texas A&M K500 Cyclotron, and the GANIL facility in Caen, France .

During a heavy-ion test, the device under test (DUT) is mounted in a vacuum chamber, biased and clocked at normal operating conditions, and exposed to a uniform beam of a selected heavy ion species at controlled LET and fluence. For FPGA testing, two complementary readout strategies are used: static testing, in which the device is irradiated with configuration memory pre-loaded and then readback to count configuration bit flips, and dynamic testing, in which the FPGA executes a test design that monitors its own outputs for errors in real time . Dynamic testing captures both configuration upsets and user-register upsets but is limited in the depth of design validation per test run; static testing provides complete configuration memory coverage but misses upsets in the user logic flip-flops.

### Proton Testing

Proton irradiation at energies of 50–200 MeV provides a practical and cost-effective alternative to heavy-ion testing for devices whose SEE cross-section vs. LET curve is required through the proton-relevant range, and for devices intended for use in proton-dominated environments (trapped proton belts, low-altitude orbits). Protons cause SEEs primarily through nuclear elastic scattering (which produces silicon or oxygen recoil nuclei with typical LET of 5–15 MeV⋅cm​2/mg) and through non-elastic nuclear reactions (which produce heavy recoil fragments with higher LET) . The resulting SEU cross-section is lower than the heavy-ion saturation cross-section by one to two orders of magnitude, but the high proton fluences available at accelerators like TRIUMF (Vancouver), IUCF (Bloomington), and the Paul Scherrer Institute (Villigen) allow statistically meaningful measurements .

Proton testing also accumulates significant TID during the SEE test (at 100 MeV, 10​11 p/cm​2 corresponds to ∼1 krad(Si)), which must be accounted for in test planning to avoid confusing TID degradation with SEE events.

### Neutron Testing

Neutron testing is the most directly relevant modality for nuclear plant applications and for ground-level qualification . The upset mechanisms for neutrons above ∼10 MeV are similar to protons, nuclear elastic scattering producing silicon recoil nuclei plus non-elastic spallation reactions producing heavier fragments, but the neutron’s lack of charge means it produces no ionization along its own track and the DUT need not be operated in vacuum. This simplifies the experimental setup considerably.

Several neutron irradiation facilities are available for FPGA testing. The Los Alamos Neutron Science Center (LANSCE) ICE House provides a quasi-monoenergetic neutron spectrum peaked at 800 MeV that is often used as a reference for atmospheric cosmic-ray testing . The Indiana University Cyclotron Facility (IUCF) produced monoenergetic 200-MeV neutrons, now primarily replaced by the Triangle Universities Nuclear Laboratory (TUNL) and the nTOF facility at CERN for intermediate energies. For reactor neutron environments, fission reactor irradiation facilities such as the MIT reactor’s fission converter beam  and the Ohio State University Research Reactor (OSURR) provide thermal-to-fast neutron spectra representative of ex-core I&C locations.

The standard approach for neutron SEE testing of FPGAs follows the JEDEC JESD89A protocol: the device is irradiated with a known neutron fluence at the test facility’s reference spectrum, and the measured upset count is scaled to the terrestrial sea-level equivalent fluence using the spectral overlap integral . Stirk et al. compared three neutron testing approaches (monoenergetic, white-spectrum, and reactor) for a complex system-on-chip and found that the accelerator white-spectrum test at LANSCE gave the closest agreement with reactor exposure results, while monoenergetic beam tests required more careful spectral correction .

### Total Ionizing Dose Testing

TID testing is performed by exposing the device to a controlled dose from a ​60Co gamma source or an X-ray irradiator at a specified dose rate, with the device biased in a representative operating condition. The JEDEC/MIL-STD-883 Method 1019 protocol specifies the irradiation conditions, readout intervals, and post-irradiation anneal procedure required for military and space qualification .

A critical subtlety in TID testing is dose-rate dependence: many MOS devices exhibit enhanced low-dose-rate sensitivity (ELDRS), in which the damage at low dose rates (<0.1 rad(Si)/s) exceeds that at high dose rates (1–100 rad(Si)/s) for the same total dose . ELDRS occurs in bipolar transistors and certain MOS linear circuits but is less pronounced in digital CMOS; it is nonetheless a qualification concern for reactor I&C systems that accumulate dose over decades at rates of millirad per hour rather than the rad-per-second rates used in accelerated laboratory tests. The MIL-STD-883 Method 1019 elevated-temperature anneal procedure is designed to provide a conservative bound on ELDRS without requiring multi-year low-dose-rate exposures .

### Ground-Level and Accelerated System Testing

For applications where the primary concern is the terrestrial cosmic-ray neutron environment, system-level testing at the actual deployment site or at a representative high-altitude location provides the most directly applicable qualification data . However, at sea-level neutron flux (∼10−2 n/cm​2/s above 10 MeV), accumulating 1010 n/cm​2, a fluence corresponding to several decades of ground-level exposure, would require ∼3×1010 seconds, making direct ground-level testing impractical for qualification purposes. Accelerated testing at mountain-top observatories (e.g., Mauna Kea, 5 km altitude, flux ∼10× sea level) or accelerator facilities bridging this gap.

Adell et al. and Artola et al. developed simulation frameworks using GEANT4 to predict ground-level neutron upset rates from device-level cross-section measurements, validating the approach against measured field failure data from commercial servers . The GEANT4 Monte Carlo toolkit , which models hadronic and electromagnetic interactions from eV to TeV energies, has become the standard simulation infrastructure for predicting SEE rates in complex radiation environments including both atmospheric and reactor neutron spectra.

## FPGA Verification Challenges in Nuclear Beam Environments

Beyond the physics of radiation damage, the experimental characterization of FPGA upset susceptibility at nuclear beam facilities presents several practical constraints that inform the design of in-beam test instruments such as those developed in Chapters [ch:digital_electronics] and [chap:fpga_benchmark].

State-volume coverage. An SEE cross-section measurement requires observing a statistically significant number of events. The cross-section for a single memory cell is of order 10−14 to 10−12 cm​2, so at a flux of 105–107 n/cm​2/s a single cell would require impractically long exposures to accumulate even one event on average. Maximising the monitored state population is therefore essential.

Observability without intrusion. Reading every flip-flop off-chip individually during irradiation is infeasible for large state populations and introduces additional wiring susceptible to radiation and noise. A compact on-chip checker that compresses the health of the state array into a small digital flag is preferable.

Configuration vs. data upsets. Any experimental design must be capable of distinguishing a user-data upset from a configuration memory upset. The two mechanisms require different mitigation strategies: data-path redundancy (DMR/TMR) addresses the former, while periodic configuration scrubbing addresses the latter.

Telemetry reliability. The UART or other communication channel used to export results is itself implemented in FPGA resources and therefore subject to the same configuration upsets. A robust transmitter design reduces the risk that the observability channel itself is corrupted during an experiment.

These constraints motivate the specific design choices made in the FFSR shift-register test structure (Chapter [ch:digital_electronics]) and the fixed-point thermal control benchmark (Chapter [chap:fpga_benchmark]), both of which employ on-chip redundancy comparison, compact telemetry encoding, and large monitored state populations to maximize the probability of detecting radiation-induced errors within the available beam time.

## Summary

This chapter has established the physical and engineering framework for the FPGA-based nuclear instrumentation systems developed in Chapters [ch:digital_electronics] and [chap:fpga_benchmark]. The three radiation environments of concern, space, nuclear plant, and terrestrial cosmic-ray, each impose distinct TID and SEE demands on digital electronics. TID damage in MOS gate oxides accumulates throughout device operation and degrades transistor performance at doses that, for control-room I&C in nuclear plants, remain well within the tolerance of modern CMOS processes. The more consequential threat is stochastic single-event effects, which occur at arbitrarily low dose rates: SEUs corrupt FPGA configuration memory silently and persistently, SEFIs can interrupt device operation, and SELs can destroy devices if power cycling is not promptly applied.

Triple modular redundancy, applied at the register and routing level within the FPGA fabric, corrects single upsets instantaneously without interrupting the safety function; periodic configuration scrubbing prevents upset accumulation that would eventually defeat TMR. The combination of these two techniques reduces the uncorrectable upset probability to levels that can be demonstrated to meet the single-failure criterion of nuclear safety standards. COTS FPGAs from the 28-nm and 16-nm Kintex-7 and UltraScale families have been qualified for radiation environments substantially more demanding than nuclear plant I&C through heavy-ion, proton, and neutron testing at national accelerator facilities, and the regulatory pathway for their use in safety-related applications is established in IAEA SSG-39 and NRC Regulatory Guide 1.152. The dissertation now turns to the perovskite materials and device investigations (Chapters [ch:CsFAPbI]–[ch:perovskite_detectors]) and thermal sensing concepts (Chapter [ch:thermal_sensing]) before returning to experimentally validate these FPGA principles: Chapter [ch:digital_electronics] presents the Flip-Flop Shift Register (FFSR) verification structure tested under fast neutron irradiation, and Chapter [chap:fpga_benchmark] extends that campaign with a computationally richer fixed-point thermal control benchmark.